HYGIENE PLAN (S1) Lab: 6.01.0  Responsible.:  Prof. Dr. Paxton   Supervision: Biological  hazards (Dr. Tragust)

What ?				When ?	  			With what ?	 		How ?		    Who ?

# Hands				after ending GMO and 		Hand disinfection solution		3 ml (30 sec)	    All!

# Disinfection		microbiological work and		Sterilium (from dispenser) and

# Cleaning		before departing the room		Bactolan (hand cream) 				    All

Work surfaces		after ending work			Bacillol plus or 			Spray (15 min)

(lab benches)								Ethanol (70%)				Wipe clean	    All

## Floors				after every possible contamination	Disinfectant				Wipe	 	    Whoever

with GMOs or microorganisms						(0.04%-0.8%)	    caused spill

Instruments /Apparatus

autoclavable		regularly (min. twice per week)	Autoclave (HG50 in Lab 6.15.0)	20 min 121 °C	    User

not autoclavable	after every possible contamination	Disinfectant				(0.04-0.8%)	    User

with GMOs or microorganisms

### Centrifuges			after every possible contamination	Disinfectant 				(0.04%-0.8%)	    User

with GMOs or microorganisms

Workbench			before and after all work		Ethanol (70%)				Wipe clean	    User

Protective clothing

Lab coats		ware when working with 		Collect together after			Sterilisation/	    (User)

Gloves			GMOs and microorganisms		possible contamination		Disinfection	    Central

(in labelled waste bags)				   processing

Waste				contaminated waste			Collect (in labelled waste bags/labelled		    User

regularly (min. twice per week)	(waste buckets and gathered in labelled containers)

Autoclave (HG50 in Lab 6.15.0)	20 min. 121°C	    User